| Models | MPS-6015W | MPS-6015L | MPS-6090-120 | MPS-6090-220 |
| --- | --- | --- | --- | --- |
| Input |  |  |  |  |
| Voltage | 220 VAC | 220 VAC | 120 VAC | 220 VAC |
| Voltage Range | + / - 15% |  |  |  |
| Frequency | 50 Hz | 60 Hz | 60 Hz | 50 Hz |
| Protection | Fuse (8A) | Fuse (8A) | Fuse (15A) | Fuse (15A) |
| Power Factor | ≥ 0.9(full load) |  |  |  |
| Output |  |  |  |  |
| Voltage | 60 VAC | 60 VAC | 60/90VAC, selectable | 60/90VAC, selectable |
| Regulation | + / - 5% |  |  |  |
| Waveform | Quasi-square |  |  |  |
| Current | 15A |  |  |  |
| Power | 900 VA | 900 VA | 900 VA | 1350 VA |
| Protection | Current limited |  |  |  |
| Short Current | 150 % of max. current rating |  |  |  |
| Efficiency | ≥ 90% |  |  |  |
| Time Delay Relay |  |  |  |  |
| Initialization | 10s (selectable, TDR is an optional feature) |  |  |  |
| Mechanical |  |  |  |  |
| Input Connection(s) | Terminal Block |  |  |  |
| Output Connection(s) | 5/8" female |  |  |  |
| Enclosure Material | Aluminum |  |  |  |
| Finish | Powder coating |  |  |  |
| Dimensions | 335 x 217 x 190mm |  |  |  |
| Environmental |  |  |  |  |
| Operating Temperature | - 40°C to 55°C |  |  |  |
| Humidity | 0 to 95% non-condensing |  |  |  |

- * Constant voltage ferroresonant transformer

- * Fully regulated, clean and reliable output AC power

- * Input and output protection, lightning surge protection

- * Current limited output and short-circuit protection

- * Automatic restart upon removal of short

- * Field selectable output voltages

- * Powder coated enclosure for outdoor applications

- * Pole & wall mount installations

- * 5/8” female output connection

- * Durable LED indicator

- * Optional Time Delay Relay (TDR)

- More voltage configurations may be available. Please download the datasheet or contact us directly for more details.

- Due to continuing improvements, the product is subject to change without prior notice.